RDKB RPI4 64bit kirkstone sanity Test Results

- Flashed GW 64bit kirkstone build in rpi4

- Goto RG console

Summary Table

| S.No | Sanity tested on | Status |
| --- | --- | --- |
| 1 | erouter0 interface got Wan IP | Pass |
| 2 | brlan0 interface got default LAN IP | Pass |
| 3 | Able to take SSH Connection | Pass |
| 4 | All CCSP process are running | Pass |
| 5 | Able to Ping www.google.com | Pass |
| 6 | All systemd services are running | Pass |
| 7 | Able to connect wireless client | Pass |
| 8 | Wireless client got IP from default pool range | Pass |
| 9 | Wireless client can access the CaptivePortal Page | Pass |
| 10 | Internet should work on Connected clients after changing the default credentials | Pass |
| 11 | Using dmcli to check the CCSP TR-181 DataModels | Pass |
| 12 | WebUI page should be loaded on Connected clients after changing the default credentials | Pass |
| 13 | WebUI page should be loaded through erouter0IP via RM | Pass |
| 14 | wlan0 and wlan1 interfaces should be UP and RUNNING | Pass |
| 15 | All ccsp logs files should be generated in /rdklogs/logs folder | Pass |
| 16 | Lan clients should be connected and receive internet | Pass |

erouter0 IPv4 address
erouter0 should have valid IP based on the wan configuration .

brlan0 IPv4 address

brlan0 IPv4 should be 10.0.0.1 ( i,e Default IP Address )

Able to take SSH from WAN IP

ssh from WAN to erouter0 IP should be successful

Internet Testing on RPI

- erouter0 Interface should have Ipv4 Address

- verify the ping www.google.com on RPI

- RPI shouldn't lose the Internet

CCSP Process Status

All ccsp process should be Up and corresponding DataModel parameters should be loaded properly.

Logs

All ccsp logs will be generated in /rdklogs/logs folder

Systemd Status

root@RaspberryPi-Gateway:~# systemctl status

● RaspberryPi-Gateway

State: starting

Jobs: 1 queued

Failed: 2 units

Since: Thu 1970-01-01 00:00:03 UTC; 54 years 0 months ago

CGroup: /

├─init.scope

│ └─ 1 /sbin/init

└─system.slice

├─CcspAdvSecuritySsp.service

│ └─ 26405 /usr/bin/CcspAdvSecuritySsp -subsys eRT.

├─CcspCrSsp.service

│ └─ 415 /usr/bin/CcspCrSsp -subsys eRT.

├─CcspEthAgent.service

│ └─ 2136 /usr/bin/CcspEthAgent -subsys eRT.

├─CcspLMLite.service

│ └─ 2239 /usr/bin/CcspLMLite -subsys eRT.

├─CcspPandMSsp.service

│ └─ 1306 /usr/bin/CcspPandMSsp -subsys eRT.

├─CcspTandDSsp.service

│ ├─  2130 /usr/bin/CcspTandDSsp -subsys eRT.

│ ├─  2398 /bin/sh /usr/ccsp/tad/self_heal_connectivity_test.sh

│ ├─  2403 /bin/sh /usr/ccsp/tad/resource_monitor.sh

│ ├─  2482 sleep 2942

│ └─ 20585 sleep 900

├─CcspTelemetry.service

│ └─ 2776 /usr/bin/telemetry2_0

├─CcspTr069PaSsp.service

│ └─ 2194 /usr/bin/CcspTr069PaSsp -subsys eRT.

├─CcspWebUI.service

│ ├─  2124 /bin/sh /lib/rdk/brlan0_check.sh

│ ├─  3983 lighttpd -f /etc/lighttpd.conf

│ └─ 26384 sleep 10

├─CcspXdnsSsp.service

│ └─ 2219 /usr/bin/CcspXdnsSsp -subsys eRT.

├─PsmSsp.service

│ └─ 426 /usr/bin/PsmSsp -subsys eRT.

├─RdkCellularManagerLite.service

│ └─ 1312 /usr/bin/cellularmanagerlite -subsys eRT.

├─RdkFwUpgradeManager.service

│ └─ 1316 /usr/bin/fwupgrademanager -subsys eRT.

├─RdkVlanManager.service

│ └─ 461 /usr/bin/VlanManager -subsys eRT.

├─RdkWanManager.service

│ ├─ 1299 /usr/rdk/wanmanager/wanmanager -subsys eRT.

│ ├─ 1505 netmonitor

│ └─ 3434 udhcpc -i erouter0 -p /tmp/udhcpc.erouter0.pid -s /etc/udhcpc.script

├─busybox-klogd.service

│ └─ 248 /sbin/klogd -n

├─busybox-syslog.service

│ └─ 249 /sbin/syslogd -n

├─dsm.service

│ └─ 398 /usr/bin/dsm

├─notifyComp.service

│ └─ 2143 /usr/bin/notify_comp -subsys eRT.

├─ntpd.service

│ └─ 2134 /usr/sbin/ntpd -N -p 0.pool.ntp.org -p 1.pool.ntp.org -p 2.pool.ntp.org -p 3.pool.ntp.org

├─onewifi.service

│ ├─ 24211 /usr/bin/OneWifi -subsys eRT.

│ └─ 24249 /var/tmp/wifidb-server /opt/secure/wifi/rdkb-wifi.db --remote=punix:/var/tmp/wifidb.sock --unixctl=/var/tmp/wifi.ctl --log-file=/dev/null --detach

├─rbus.service

│ └─ 284 /usr/bin/rtrouted

├─rbus_log.service

│ ├─   399 /bin/sh /usr/bin/rbus_log_capture.sh

│ └─ 26398 sleep 10

├─rbus_session_mgr.service

│ └─ 402 /usr/bin/rbus_session_mgr

├─rdkbLogMonitor.service

│ ├─   974 /bin/sh /rdklogger/rdkbLogMonitor.sh

│ └─ 26328 sleep 10

├─snmpSubAgent.service

│ └─ 2144 /usr/bin/snmp_subagent "&"

├─snmpd.service

│ └─ 285 /usr/sbin/snmpd -f -C -c /usr/ccsp/snmp/snmpd.conf -M /usr/share/snmp/mibs -Le

├─system-getty.slice

│ └─getty@tty1.service

│   └─ 259 /sbin/agetty -o "-p -- \\u" --noclear - linux

├─system-serial\x2dgetty.slice

│ └─serial-getty@ttyS0.service

│   ├─ 283 /bin/login -f

│   └─ 758 -sh

├─systemd-journald.service

│ └─ 155 /lib/systemd/systemd-journald

├─systemd-udevd.service

│ └─ 175 /lib/systemd/systemd-udevd

├─usp-pa.service

│ └─ 3427 /usr/bin/UspPa -v1 --resetfile /etc/usp-pa/usp_factory_reset.conf --truststore /etc/usp-pa/usp_truststore.pem --interface erouter0 --log syslog --dbfile /nvram/usp-pa.db

├─utopia.service

│ ├─   347 klogd -c 6

│ ├─   350 syslogd -l 6

│ ├─   377 syseventd

│ ├─   393 /usr/bin/syseventd_fork_helper 12

│ ├─   838 crond -l 9

│ ├─  1013 trigger

│ ├─  1216 dropbear -R -E -a -r /tmp/.dropbear/dropcfg1949 -r /tmp/.dropbear/dropcfg2949 -p "[]:22" -P /var/run/dropbear.pid -B

│ ├─  1683 IGD brlan0

│ ├─  3508 nfq_handler 4

│ ├─  7328 dropbear -R -E -a -r /tmp/.dropbear/dropcfg1949 -r /tmp/.dropbear/dropcfg2949 -p "[]:22" -P /var/run/dropbear.pid -B

│ ├─  7339 -sh

│ ├─ 12629 zebra -d -f /var/zebra.conf -P 0

│ ├─ 12940 dnsmasq -P 4096 -C /var/dnsmasq.conf

│ ├─ 26408 systemctl status

│ └─ 26409 /bin/cat

└─webpabroadband.service

└─ 3040 /usr/bin/webpa "&"

WebUI Test on RPI

WebUI should be loaded in Webbrowser by using erouter0 IPv4 Address(i,e erouter0ip:80) and login should be successful

WiFi Status

root@RaspberryPi-Gateway:~# brctl show

bridge name	bridge id		STP enabled	interfaces

br0		8000.000000000000	no

br106		8000.000000000000	no

br403		8000.000000000000	no

brebhaul		8000.000000000000	no

brlan0		8000.70886b8104e6	no		eth1

Captive Portal