Minimal Functionality for BPI-R4 Beta 64b 20250130114818 GW Build

| S.No | Tested On | Status | Remarks |
| --- | --- | --- | --- |
| 1 | SNMP | Pass | SNMP set and get are working fine(checked for ssid password) |
| 2 | Local WebUI | Pass | All items are dispayed in webui are working as expected, except the below: - Gateway > Connection > RDKM > Cable Modem > Core Version is empty - Gateway > Connection > RDKM > Cable Modem > Product Type is XB3 - Gateway > Connection > RDKM > Downstream is empty - Gateway > Connection > RDKM > Upstream is empty - Gateway > Connection > RDKM > CM Error Codewords is empty - Gateway > Hardware > System Hardware > Processor Speed MHz (no data) - Gateway > Connection > RDKM (eMTA MAC, CM MAC empty, remaing are updated) - Gateway > Software ( Packet Cable is empty ) - Gateway > Hardware > LAN Ethernet ( Page is Empty) - WPS option is not available in webui – ticket rasied |
| 3 | WAN connectivity | Pass – Ipv4 Pass - IPv6 | Able to ping both IPv4 and IPv6 Able to connect and able to connect to internet. EthWan not available. |
| 4 | SSH | Pass | Able to connect SSH and copy the files from/to rpi4. |
| 5 | TR-069 | Pass | Able to see the device as online in ACS server. Able to get the param(Device.Time.LocalTimeZone) and (Device.X_CISCO_COM_DeviceControl.LanManagementEntry.1.LanMode)in ACS server(reflecting in 2-3mins). Able to set the param from ACS server(reflecting in 5mins of time after refreshing in server again). |
| 6 | Self Heal | Pass | Able to get the logs within the specified time |
| 7 | WebPA | Pass | Able to set and retrieve the same in get params(Checked for 5G SSID and 2G SSID). |
| 8 | Parental control | Pass - Sites, Devices & Services Pass- Reports | Able to block the sites, keywords, devices and allow the specified devices. Able to get the reports for the managed sites,devices,services. |
| 9 | Firewall | Pass | Able to get the iptables rules(PORT_SCAN, FRAG_DROP, DOS) after setting the DM’s |
| 10 | Firmware upgrade | Pass | Able to see upgraded image after restarting the swupdate.service |
| 11 | Wi-Fi | Pass – Ipv4 Pass – Ipv6 Pass – 2G & 5G Partially pass - 6G - connectivity issue while reconnecting in clients | Able to get the Ipv4 & Ipv6 address in connected clients(WAN clients), captive portal page is opening. |
| 12 | Wifi data Persistence after reboot | Pass | Able to observe the changed SSID’s. Note:- After reboot wifi data is persistent. |
| 13 | LAN clients | Pass – Ipv4 Pass --IPv6 | Lan clients are getting the internet and captive portal page is opening. |
| 14 | DAC | Pass -DobbyTool Pass -UspPa | Working via DobbyTool. List is happening when we run the container using DobbyTool in RPI4. Able to start DobbyTool via UspPa, Able to see DobbyTool list. |
| 15 | WebConfig | Pass | Able to change wifi ssid and password for Onewifi, PAM, DMZ, Able to see logs for Telemetry2 multi-profiles. |
| 16 | Telemetry 2.0 | Pass - Single profile through XCONF Pass - Multi profile through XCONF | Using DMCLI set, we can able to get the Cjson report in the telemetry logs Able to get the logs via xconf set profiles Single profile via Xconf is working and able to see CJSON getting generated. Multiprofile via xconf is working and cjson is also getting generated. |
| 17 | Bridge mode | Partially Pass ( At first boot worked, after reboot client getting ip in 10.0.0.1 series ) | Router to Bridge Mode: Private wifi’s are down & brlan0 losses it’s IP & lan0 is getting the IP. Lan client is getting erouter0 Ip and able to open the webui with 10.0.0.1 Bridge Mode to Router: Brlan0 it retreiving it’s address, webui is opening, Lan client is getting brlan0 IP., Private wifi’s are up. |
| 18 | RFC | Pass | Able to get the expected tr-181 params(which are set in xconf) in RPI4 device and the /tmp/rfc_configdata.txt is getting created. |
| 19 | Log upload | Pass | Able to see the *tar.gz under /rdklogs/logs/PreviousLogs_backup file. Able to see the *.tgz logs under /rdklogs/logs/62:D7:12:07:17:91-Logs-02-03-25-02-30PM.tgz 06724900d523_TELE_TEST7302-03-2025-01-33PM.json |
| 20 | Log rotation | Pass | Tar file is observed under “/nvram2/logs” 62:D7:12:07:17:91-Logs-02-03-25-02-30PM.tgz |
| 21 | USP Agent/TR-369 | Pass | Able to get & set values through UspPa(Dbus changes are present) Able to set/get from oktopus controller |
| 22 | Cellular Mode | Pass | Able to connect device using cellular wan. Eth to cellular and cellular to Eth is working. Able to get internet in BPI and connected clients. |
| 23 | CPU Proc Analyser | Pass | Able to see wifi spectrum for 2g and 5g in webui |
| 24 | PopulateSDK | Pass | By using script able to generate binary and execute it |
| 25 | eSDK | Partially Pass(Need to confirm those TC's are enough/not) | For eSDK: Tested below: devtool edit-recipe <recipe_name> devtool build <recipe_name> devtool add <recipe_name> <source URL> |
| 26 | WiFi 6E | Pass | Able to connect 6ghz and able to browse Able to change 6g configurations via dmcli / WebUI successfully |
| 27 | Test and Diagnosis | Pass | Up and running, IP ping and traceroute is working |
| 28 | Crash Upload | Pass | Minidumps folder got created and core_log.txt is getting updated |
| 29 | Factory reset | Pass | SSID & WEBUI passwords are changed to default |

For the below Image we tested:

root@Filogic-GW:~# cat /version.txt

imagename:rdkb-generic-broadband-image_rdk-next_20250130114818

BRANCH=rdk-next

YOCTO_VERSION=kirkstone

VERSION=6.1.01.30.25

SPIN=0

BUILD_TIME="2025-01-30 11:48:18"

JENKINS_JOB=Default

JENKINS_BUILD_NUMBER=0

Generated on Thu Jan 30  11:48:18 UTC 2025

root@Filogic-GW:~# uname -a

Linux Filogic-GW 5.4.271-yocto-standard #1 SMP Wed Mar 6 14:36:10 UTC 2024 aarch64 GNU/Linux

root@Filogic-GW:~#