Features Support in BPI

| S.No | Supported Features |
| --- | --- |
| 1 | WAN Connectivity |
| 2 | WiFi Connectivity |
| 3 | LAN Connectivity |
| 3 | Firewall |
| 4 | OneWifi with single radio (i,e 2GHz) |
| 5 | local webui |
| 6 | Log Rotation |
| 7 | Webconfig |
| 8 | RFC |
| 9 | Telemetry 2.0 |
| 10 | WebPA |
| 11 | USP PA |
| 12 | cpuproc analyzer |
| 13 | Cellular use cases support - Qualcomm/MTK modems |

RDKB BPI4 Kirkstone Build Test Results

Image : rdkb-generic-broadband-image_rdk-next_20241028074211

| S.No | Sanity tested on | Status | Comment |
| --- | --- | --- | --- |
| 1 | erouter0 interface got Wan IPv4 and IPv6 | Pass |  |
| 2 | brlan0 interface got default LAN IP & Ipv6 | Pass |  |
| 3 | Able to take SSH Connection | Pass |  |
| 4 | All CCSP process are running | Pass |  |
| 5 | Able to Ping www.google.com & ping -6 www.google.com in Gateway (i,e erouter0) | Pass |  |
| 6 | All systemd services are running | Pass |  |
| 7 | Able to connect LAN client | Pass |  |
| 8 | LAN client got IP from default pool range | Ipv4:Pass Ipv6:Pass |  |
| 9 | LAN client should open with CaptivePortal Page | Pass |  |
| 10 | Able to connect wireless client | Pass |  |
| 11 | Wireless client got IP from default pool range(i,e 2G) | Ipv4:Pass Ipv6:Pass |  |
| 12 | Wireless client can access the CaptivePortal Page(i,e 2G) | Pass |  |
| 13 | Internet should work on Connected clients after changing the default credentials | Pass |  |
| 14 | Using dmcli to check the CCSP TR-181 DataModels | Pass |  |
| 15 | WebUI page should be loaded on Connected clients after changing the default credentials | Pass |  |
| 16 | WebUI page should be loaded through erouter0IP via RM | Pass |  |
| 17 | wifi0 interface should be UP and RUNNING | Pass |  |
| 18 | All ccsp logs files should be generated in /rdklogs/logs folder | Pass |  |
| 19 | WebPa get and set should work, query device. | Pass |  |
| 20 | LogUpload/Logrotation should work | Pass |  |
| 21 | RFC should work form xconf | Pass |  |
| 22 | Webconfig Feature : WiFi, LAN, DMZ, Portmapping | Pass |  |
| 23 | Usppa get and set are working through DM’s | Pass |  |
| 24 | Cellular Manager WanMode | Pass |  |
| 25 | Factory Reset (Failed for WiFi known issue) | Pass |  |
| 26 | Telemetry 2.0 Feature | Pass |  |

root@Filogic-GW:~# cat /version.txt

imagename:rdkb-generic-broadband-image_rdk-next_20241028074211

BRANCH=rdk-next

YOCTO_VERSION=kirkstone

VERSION=6.1.10.28.24

SPIN=0

BUILD_TIME="2024-10-28 07:42:11"

Generated on Mon Oct 28  07:42:11 UTC 2024

root@Filogic-GW:~#  uname -a

Linux Filogic-GW 5.4.271-yocto-standard #1 SMP Wed Mar 6 14:36:10 UTC 2024 aarch64 GNU/Linux