RDKB RPI4 32b Dunfell GW Build Sanity Test Results

- Flashed GW 32b dunfell build in rpi4

- Goto RG console

Summary Table

| S.No | Sanity tested on | Status |
| --- | --- | --- |
| 1 | erouter0 interface got Wan IP | Pass |
| 2 | brlan0 interface got default LAN IP | Pass |
| 3 | Able to take SSH Connection | Pass |
| 4 | All CCSP process are running | Pass |
| 5 | Able to Ping www.google.com | Pass |
| 6 | All systemd services are running | Pass |
| 7 | Able to connect wireless client | Pass |
| 8 | Wireless client got IP from default pool range | Pass |
| 9 | Wireless client can access the CaptivePortal Page | Pass |
| 10 | Internet should work on Connected clients after changing the default credentials | Pass |
| 11 | Using dmcli to check the CCSP TR-181 DataModels | Pass |
| 12 | WebUI page should be loaded on Connected clients after changing the default credentials | Pass |
| 13 | WebUI page should be loaded through erouter0IP via RM | Pass |
| 14 | wlan0 and wlan1 interfaces should be UP and RUNNING | Pass |
| 15 | All ccsp logs files should be generated in /rdklogs/logs folder | Pass |
| 16 | Lan clients should be connected and receive internet | Pass |

erouter0 IPv4 address
erouter0 should have valid IP based on the wan configuration .

brlan0 IPv4 address

brlan0 IPv4 should be 10.0.0.1 ( i,e Default IP Address )

Able to take SSH from WAN IP

ssh from WAN to erouter0 IP should be successful

Internet Testing on RPI

- erouter0 Interface should have Ipv4 Address

- verify the ping www.google.com on RPI

- RPI shouldn't lose the Internet

CCSP Process Status

All ccsp process should be Up and corresponding DataModel parameters should be loaded properly.

Logs

All ccsp logs will be generated in /rdklogs/logs folder

Systemd Status

root@RaspberryPi-Gateway:/rdklogs/logs# systemctl status

● RaspberryPi-Gateway

State: degraded

Jobs: 1 queued

Failed: 1 units

Since: Thu 1970-01-01 00:00:03 UTC; 54 years 1 months ago

CGroup: /

├─user.slice

│ └─user-0.slice

│   ├─session-c1.scope

│   │ ├─ 457 /bin/login -f

│   │ └─1012 -sh

│   └─user@0.service …

│     └─init.scope

│       ├─907 /lib/systemd/systemd --user

│       └─910 (sd-pam)

├─init.scope

│ └─1 /sbin/init

└─system.slice

├─CcspTr069PaSsp.service

│ └─2261 /usr/bin/CcspTr069PaSsp -subsys eRT.

├─rbus_session_mgr.service

│ └─452 /usr/bin/rbus_session_mgr

├─notifyComp.service

│ └─2171 /usr/bin/notify_comp -subsys eRT.

├─CcspTandDSsp.service

│ ├─  738 sleep 3600

│ ├─ 2147 /usr/bin/CcspTandDSsp -subsys eRT.

│ ├─ 2402 /bin/sh /usr/ccsp/tad/self_heal_connectivity_test.sh

│ ├─ 2405 /bin/sh /usr/ccsp/tad/resource_monitor.sh

│ └─21535 sleep 900

├─CcspAdvSecuritySsp.service

│ └─23643 sleep 30

├─OvsAgent.service

│ └─613 /usr/bin/OvsAgent

├─snmpSubAgent.service

│ └─2180 /usr/bin/snmp_subagent &

├─dbus.service

│ └─239 /usr/bin/dbus-daemon --system --address=systemd: --nofork --nopidfile --systemd-activation --syslog-only

├─rbus.service

│ └─280 /usr/bin/rtrouted

├─dsm.service

│ └─441 /usr/bin/dsm

├─CcspXdnsSsp.service

│ └─2278 /usr/bin/CcspXdnsSsp -subsys eRT.

├─parodus.service

│ └─2675 /usr/bin/parodus --hw-model=RPI --hw-serial-number=10000000e534279c --hw-manufacturer=Raspberry --hw-last-reboot-reason=factory-reset --fw-name=rdkb-generic-broadband-image_rdk-next_20240219124432 --boot-time=1708341203 --hw-mac=DC:A6:32:1B:82:45 --webpa-ping-time=180 --webpa-interface-used=erouter0 --webpa-url=http://webpa.rdkcentral.com:8080 --webpa-backoff-max=9 --parodus-local-url=tcp://127.0.0.1:6666 --partner-id=comcast --ssl-cert-path=/etc/ssl/certs/ca-certificates.crt --force-ipv4

├─utopia.service

│ ├─  343 klogd -c 6

│ ├─  346 syslogd -l 6

│ ├─  423 syseventd

│ ├─  439 /usr/bin/syseventd_fork_helper 12

│ ├─  752 crond -l 9

│ ├─ 1230 trigger

│ ├─ 1312 dropbear -R -E -a -r /tmp/.dropbear/dropcfg11119 -r /tmp/.dropbear/dropcfg21119 -p []:22 -P /var/run/dropbear.pid -B

│ ├─ 3669 nfq_handler 4

│ ├─ 4024 dropbear -R -E -a -r /tmp/.dropbear/dropcfg11119 -r /tmp/.dropbear/dropcfg21119 -p []:22 -P /var/run/dropbear.pid -B

│ ├─ 4034 -sh

│ ├─10527 dnsmasq -P 4096 -C /var/dnsmasq.conf

│ ├─10583 zebra -d -f /var/zebra.conf -P 0

│ ├─23798 systemctl status

│ └─23799 /bin/cat

├─CcspTelemetry.service

│ └─2887 /usr/bin/telemetry2_0

├─rbus_log.service

│ ├─  450 /bin/sh /usr/bin/rbus_log_capture.sh

│ └─23509 sleep 10

├─system-getty.slice

│ └─getty@tty1.service

│   └─445 /sbin/agetty -o -p -- \u --noclear tty1 linux

├─RdkCellularManager.service

│ └─886 /usr/bin/cellularmanager -subsys eRT.

├─CcspLMLite.service

│ └─2287 /usr/bin/CcspLMLite -subsys eRT.

├─CcspPandMSsp.service

│ └─1054 /usr/bin/CcspPandMSsp -subsys eRT.

├─ModemManager.service

│ └─235 /usr/sbin/ModemManager

├─systemd-logind.service

│ └─303 /lib/systemd/systemd-logind

├─rdkbLogMonitor.service

│ ├─  938 /bin/sh /rdklogger/rdkbLogMonitor.sh

│ └─23239 sleep 60

├─PsmSsp.service

│ └─476 /usr/bin/PsmSsp -subsys eRT.

├─systemd-udevd.service

│ └─180 /lib/systemd/systemd-udevd

├─webpabroadband.service

│ └─3084 /usr/bin/webpa &

├─CcspEthAgent.service

│ └─2154 /usr/bin/CcspEthAgent -subsys eRT.

├─busybox-syslog.service

│ └─238 /sbin/syslogd -n

├─dobby.service

│ └─246 /usr/sbin/DobbyDaemon --nofork --noconsole --journald

├─snmpd.service

│ └─279 /usr/sbin/snmpd -f -C -c /usr/ccsp/snmp/snmpd.conf -M /usr/share/snmp/mibs -Le

├─RdkWanManager.service

│ ├─ 870 /usr/rdk/wanmanager/wanmanager -subsys eRT.

│ ├─1037 netmonitor

│ └─3633 udhcpc -i erouter0 -p /tmp/udhcpc.erouter0.pid -s /etc/udhcpc.script

├─systemd-journald.service

│ └─160 /lib/systemd/systemd-journald

├─busybox-klogd.service

│ └─237 /sbin/klogd -n

├─RdkFwUpgradeManager.service

│ └─890 /usr/bin/fwupgrademanager -subsys eRT.

├─OvsAgent_ovsdb-server.service

│ ├─557 ovsdb-server: monitoring pid 558 (healthy)

│ ├─558 /usr/sbin/ovsdb-server --remote=punix:/var/run/openvswitch/db.sock --remote=db:Open_vSwitch,Open_vSwitch,manager_options --private-key=db:Open_vSwitch,SSL,private_key --certificate=db:Open_vSwitch,SSL,certificate --ca-cert=db:Open_vSwitch,SSL,ca_cert --pidfile=/var/run/openvswitch/ovsdb-server.pid --detach --monitor --log-file=/dev/null --unixctl=/var/run/openvswitch/ovsdb-server.ctl /tmp/etc/openvswitch/opensync_conf.db /tmp/etc/openvswitch/rdkb-wifi.db

│ └─580 ovs-vswitchd --pidfile=/var/run/openvswitch/ovs-vswitchd.pid --log-file=/dev/null --detach

├─ntpd.service

│ └─2161 /usr/sbin/ntpd -N -p 0.pool.ntp.org -p 1.pool.ntp.org -p 2.pool.ntp.org -p 3.pool.ntp.org

├─onewifi.service

│ └─3051 /usr/bin/OneWifi -subsys eRT.

├─usp-pa.service

│ └─3622 /usr/bin/UspPa -v1 --resetfile /etc/usp-pa/usp_factory_reset.conf --truststore /etc/usp-pa/usp_truststore.pem --interface erouter0 --log syslog --dbfile /nvram/usp-pa.db

├─meshAgent.service

│ └─4754 /usr/bin/meshAgent -subsys eRT.

├─CcspWebUI.service

│ ├─ 2153 /bin/sh /lib/rdk/brlan0_check.sh

│ ├─ 4164 lighttpd -f /etc/lighttpd.conf

│ └─23722 sleep 10

├─RdkVlanManager.service

│ └─527 /usr/bin/VlanManager -subsys eRT.

└─CcspCrSsp.service

└─467 /usr/bin/CcspCrSsp -subsys eRT.

WebUI Test on RPI

WebUI should be loaded in Webbrowser by using erouter0 IPv4 Address(i,e erouter0ip:80) and login should be successful

WiFi Status

Brctl show

root@RaspberryPi-Gateway:~# iw dev

phy#2

Interface wlan2

ifindex 20

wdev 0x200000001

addr 9e:a2:f4:d5:ed:96

type managed

txpower 12.00 dBm

phy#1

Interface wlan1

ifindex 19

wdev 0x100000001

addr 9c:a2:f4:d5:ed:96

ssid testwifi5g

type AP

txpower 12.00 dBm

phy#0

Interface wlan0

ifindex 18

wdev 0x1

addr dc:a6:32:1b:82:47

ssid testwifi2g

type AP

channel 1 (2412 MHz), width: 20 MHz, center1: 2412 MHz

txpower 31.00 dBm

Captive Portal